Pi 5 Home Network — Concepts Guide

Everything you learned building your Pi-hole deployment

# 1. What is DNS?

DNS stands for Domain Name System. Think of it like a phonebook for the internet.

When you type a website address, DNS translates it to an IP address your computer can actually use:

You type   → google.com

DNS says   → 142.250.80.46

Browser    → goes to that IP address

Without DNS you'd have to memorize IP addresses for every website. Nobody wants that.

# 2. How Internet Traffic Flows Through Your Network

## Without Pi-hole (before):

You type youtube.com

Device asks T-Mobile DNS → gets the IP → loads YouTube

YouTube loads an ad from doubleclick.net

Device asks T-Mobile DNS → gets the IP → ad loads 😬

## With Pi-hole (now):

You type youtube.com

Device asks Pi-hole → not an ad domain → asks Cloudflare → gets IP → loads YouTube ✅

YouTube tries to load ad from doubleclick.net

Device asks Pi-hole → AD DOMAIN → returns nothing → ad never loads ✅

# 3. What is Cloudflare?

Cloudflare is a company that runs one of the fastest and most private DNS servers in the world. Their DNS address is 1.1.1.1.

Every DNS request has to go somewhere. By default it goes to your ISP (T-Mobile). You chose Cloudflare instead because:

- T-Mobile DNS: slow, logs everything you visit, sells that data

- Google DNS (8.8.8.8): fast but Google logs everything too

- Cloudflare (1.1.1.1): fastest DNS on the planet, privacy focused, doesn't log or sell your data ✅

## How it fits in your network:

Your devices

→ ask Pi-hole first (192.168.12.240)

Pi-hole blocks ad domains

forwards everything else to...

→ Cloudflare (1.1.1.1)

looks up the real IP address

sends it back to Pi-hole

Pi-hole sends it to your device

Plain English: Pi-hole is the bouncer. Cloudflare is the phonebook. They work together.

# 4. What Does the Router Do?

The router is the main hub of your whole network. Everything in your house that connects to the internet goes THROUGH the router.

## Three main jobs:

- Traffic cop: manages multiple devices using the internet at the same time without them crashing into each other

- Address assigner (DHCP): automatically assigns IP addresses to every device that joins your WiFi

- Gateway to internet (NAT): translates between your local device addresses and the public internet

## Your specific setup:

T-Mobile Gateway (192.168.12.1)

→ connects to T-Mobile cell towers

→ brings internet into your house

→ assigns IPs to all devices

Pi-hole (192.168.12.240)

→ sits inside your network

→ intercepts DNS requests

→ router doesn't even know it's blocking ads 😄

Plain English: Router = the post office. Pi-hole = the mail sorter that throws away junk mail before it reaches you.

# 5. What Does Pi-hole Do?

Pi-hole is a DNS server that runs on YOUR Raspberry Pi inside YOUR house. Instead of your devices asking T-Mobile or Google to look up websites, they ask Pi-hole first.

## The blocklist:

During installation you added StevenBlack's Unified Hosts List — 82,624 known ad and tracking domains. Pi-hole checks every single DNS request against this list:

Known ad domain?  → blocked 🚫

Unknown domain?   → forwarded to Cloudflare ✅

## Why it works on ALL devices:

- Normal ad blockers: browser extension only, doesn't work in apps

- Pi-hole: works at the network level, before requests even leave your house, works on every device using it as DNS

## The one limitation:

YouTube, Instagram, and Facebook serve ads from their OWN servers on the same domain as actual content. Pi-hole can't block those without breaking the entire app. That's why ads still show in those apps on mobile.

# 6. Why Are Ads Blocked on Laptop YouTube But Not Mobile?

## Your MacBook:

DNS set to → 192.168.12.240 (Pi-hole) ✅

Using YouTube in browser → separate ad requests

Pi-hole blocks those requests → no ads ✅

## Your iPhone YouTube app:

DNS set to → 192.168.12.240 (Pi-hole) ✅

But YouTube APP → Google serves ads from

same servers as the video

Pi-hole can't block without breaking app ❌

Think of it like this: Website ads are delivered by a UPS truck — Pi-hole blocks the truck. YouTube app ads are baked INTO the package — Pi-hole can't open the package without breaking everything.

## Solutions for YouTube app ads:

- YouTube Premium ($14/mo) — no ads anywhere

- Use YouTube in Safari instead of the app + install AdGuard extension

- Install uBlock Origin on desktop browser — Pi-hole + uBlock = ultimate combo

# 7. Does Pi-hole Make My Internet Faster?

Kind of — but here's the honest answer:

Cloudflare DNS  → faster DNS lookups ✅

Pi-hole         → adds tiny processing delay ❌

Net result      → basically the same speed

DNS speed only affects the lookup phase (20-50ms normally). After DNS resolves, downloading pages, watching YouTube, and streaming Netflix are not affected by Pi-hole at all.

## What Pi-hole actually improves:

- Ads don't load at all → pages feel cleaner

- Trackers can't follow you across sites → better privacy

- Less data collection by advertisers

- Faster page loads on ad-heavy sites (fewer requests to make)

# 8. Why Does DNS Change on Each Network?

When you connect to a new WiFi network in DHCP mode, the router of THAT network automatically assigns you an IP address, subnet mask, gateway, and DNS server. Every router has its own DNS.

Your home T-Mobile router  → DNS: 192.168.12.1

Suju's Coffee router       → DNS: their ISP's server

Your iPhone hotspot        → DNS: different again

Think of it like this: every WiFi network is a different city. Each city has its own phonebook (DNS). When you drive to a new city, DHCP automatically hands you that city's phonebook. You don't bring your home phonebook with you.

## Why Static IP breaks at Suju's:

At home you manually set DNS to 192.168.12.240 (your Pi). At Suju's that IP doesn't exist — it's your home Pi sitting in your house. Suju's network has never heard of it. The phone tries to ask Pi-hole, Pi-hole isn't there, DNS fails, WiFi appears broken.

## DHCP vs Static:

DHCP   → 'use whatever DNS this network gives me'

works everywhere ✅

but DNS changes per network

Static → 'ONLY use Pi-hole'

great at home ✅

breaks everywhere else ❌

# 9. Your Home Network Setup

Router: T-Mobile KVD21 Gateway

Router IP: 192.168.12.1

Pi hostname: retropi

Pi IP: 192.168.12.240

Pi-hole DNS: 192.168.12.240

Upstream DNS: Cloudflare (1.1.1.1)

Blocklist: StevenBlack Unified Hosts — 82,624 domains

Devices using Pi-hole: MacBook, iPhone, Wife's OnePlus

SSH command: ssh jovi@retropi.local

Pi-hole dashboard: http://192.168.12.240/admin

Pi-hole password: s2e2o4oo

# 10. Key Commands to Know

## SSH into Pi:

ssh jovi@retropi.local

## Update Pi system:

sudo apt update && sudo apt upgrade -y

## Restart Pi-hole:

pihole restartdns

## Check Pi-hole status:

pihole status

You built all of this from scratch. That's not nothing.